ExpertAmateur.com  |  Blog Post  |  March 2026

# The Apartment Energy Stack:
Balcony Solar + Battery + EMS + Presence Detection

12 min read  •  Beginner-friendly  •  Home Assistant

Most of the coverage you'll read about balcony solar stops at the panel. Plug it in, save some money, done. That's fine — but it's leaving a lot of efficiency on the table. This post is about what happens when you treat a small balcony system the way you'd treat a serious off-grid installation: with a battery, an Energy Management System, and presence detection that makes the whole thing context-aware.

The result is a system that's smarter than almost any commercial smart home product — and you can build it for a fraction of the cost of a rooftop installation.

The insight: the ratio of smart management to available energy matters just as much on a small system as a large one. A well-tuned EMS on a 400W balcony setup can outperform a passive 800W setup in real-world energy savings.

## Why Passive Balcony Solar Underperforms

A bare balcony solar panel — panel, microinverter, wall outlet — is a passive device. It produces power when the sun shines and feeds it into your home wiring. Whatever appliances happen to be running at that moment benefit. The rest gets exported to the grid (at low or zero credit, depending on your utility) or simply wasted.

The problem is alignment. Peak solar production is midday. Peak apartment energy use is morning and evening. Without any storage or smart management, you might capture 50–65% of what your panel produces in a way that actually offsets your bill. The rest just doesn't match up with when you need it.

Add a battery — even a modest portable power station like a Bluetti or EcoFlow — and suddenly you can store what the panel produces during the day and deploy it when you actually need it. Add an EMS and presence detection, and the system decides intelligently when to charge, when to discharge, and which loads to power from stored solar versus the grid.

The payback math changes significantly. A passive $1,000 system saving $120/year pays back in 8+ years. The same system with smart management capturing 20% more usable energy saves closer to $150/year — and that gap compounds as electricity rates rise.

## The Four-Layer Stack

Think of this as a layered system where each layer makes the one below it more effective:

Layer 1 — The Panel

One or two 400W bifacial panels on your balcony, railing, or patio. This is your generation source. Without the layers above it, this is all you have: passive power when the sun shines.

Layer 2 — The Battery/Inverter

A portable power station (LiFePO4 chemistry — safer, longer cycle life) in the 1–3 kWh range is the right fit for most apartments. This stores excess daytime production and makes it dispatchable — you decide when it's used, not the sun. Some units like the EcoFlow Delta Pro can also act as an uninterruptible power supply for your critical loads.

At this layer you've transformed a passive system into an active one. But without Layer 3, you're still just charging and discharging manually or on a fixed schedule.

Layer 3 — The Energy Management System (EMS)

This is Home Assistant running on a Raspberry Pi, with a few sensors and automations. The EMS watches your solar production in real time, monitors your battery state of charge, knows your electricity rate schedule (if your utility has time-of-use pricing), and makes decisions accordingly.

Key automations at this layer:

- Solar threshold charging: When panel output exceeds baseline loads, direct excess to battery

- Load shifting: Run high-draw appliances (dishwasher, laundry, hair dryer) automatically when solar is producing

- TOU optimization: Draw from battery during peak-rate hours, charge from grid during off-peak if needed

- Battery protection: Never discharge below 20% SoC; prioritize overnight reserve based on tomorrow's forecast

The EMS is what separates a smart system from a dumb one. Same hardware, meaningfully better outcomes.

Layer 4 — Presence Detection

This is where the system becomes genuinely intelligent rather than just automated. Presence detection tells your EMS whether anyone is home — and changes the logic accordingly.

In Home Assistant, presence detection can come from phone GPS/WiFi, a door sensor, a motion sensor, or a combination. Once the EMS knows occupancy state, it can make decisions that a schedule-based system never could:

- Nobody home, solar producing: Don't run the AC. Charge the battery instead. No point cooling an empty apartment.

- Someone arriving in 15 minutes, battery charged: Pre-cool the apartment using stored solar before they walk in

- Everyone just left: Cut the window AC immediately regardless of schedule or thermostat setting

- Peak solar + someone home: Run the AC now from direct solar, not from the battery or grid at peak rates

- Guest expected, cloudy day: Draw down battery to cover comfort loads; flag that tomorrow's solar should recharge

Window AC units are typically 400–600W — often the single largest load in an apartment system. Getting AC management right can be the difference between a battery that lasts all evening and one that's depleted by 4pm.

## The Window AC Problem — and the Solution

Most window AC units have no smart interface at all. They're not WiFi-enabled, they don't have APIs, they just have a remote and a thermostat dial. That's fine — Home Assistant doesn't need them to be smart. You have a few options:

- Smart plug with energy monitoring (Kasa, Shelly) — gives you on/off control and real-time wattage. Simple and effective.

- IR blaster (Sensibo, Cielo Breez, or a DIY Broadlink setup) — emulates your remote, gives you full control over temperature setpoints, modes, and fan speed

- Upgrade to a mini-split if your lease allows — these are inherently smarter and vastly more efficient per BTU

With a smart plug or IR blaster integrated into Home Assistant, your window AC becomes a fully managed load. The EMS can turn it on when solar is producing, off when you leave, and set it to a specific temperature based on your arrival time — all automatically.

A "smart" thermostat like a Nest or Ecobee does some of this, but it doesn't know about your solar production or battery state. It's optimizing for comfort and schedule. Your EMS optimizes for all three: comfort, schedule, and energy source. That's a fundamentally more capable system.

## Real-World Example: A Tuesday in Your Apartment

Here's what a well-tuned four-layer system looks like in practice:

- 7:00am — You leave for work. Presence detection sees everyone out. EMS turns off AC, reduces standby loads to minimum.

- 9:00am — Solar production starts. Battery begins charging from panel output.

- 11:30am — Battery hits 80% SoC. EMS shifts excess solar to water heater timer (if applicable) or allows grid export.

- 12:00pm — Peak solar. EMS checks: is anyone home? No. Continues banking energy.

- 5:15pm — Your phone's GPS shows you 10 minutes out. EMS starts AC using battery power. Apartment is comfortable when you arrive.

- 5:30pm — You're home. AC runs from battery (charged by today's solar) through the evening peak-rate window.

- 9:00pm — Battery at 30% SoC. EMS cuts AC to minimum, switches non-critical loads back to grid for overnight.

- Next morning — Battery reserve ready for tomorrow's cycle.

None of that required any manual intervention. It just happened, because the system knows what's producing, what's stored, and who's home.

## Hardware Shopping List (Apartment Starter Stack)

- Solar panels: 1–2 x 400W bifacial panels (~$150–300 each)

- Microinverter: Hoymiles HM-400 or APsystems EZ1 (~$80–120)

- Portable battery: EcoFlow Delta 2 (1kWh, ~$500–600 on sale) or Bluetti AC180 (~$500)

- Raspberry Pi 4 (2GB) for Home Assistant (~$45 + SD card)

- Smart plugs with energy monitoring: Shelly Plug S or Kasa EP25 (~$15–20 each)

- IR blaster for window AC: Sensibo Air or Cielo Breez (~$50–100) — optional but powerful

- Presence detection: your existing iPhone or Android (Home Assistant Companion app, free)

Total system cost: roughly $1,000–1,500 depending on what you already have. That's a complete, genuinely intelligent energy management system — not just a panel on a railing.

## The ExpertAmateur Energy Ladder

This four-layer apartment stack is Tier 1 of the ExpertAmateur approach to energy independence. Everything you build here scales directly:

- The Home Assistant skills you develop managing a 400W system are identical to managing a 4,000W system

- The EMS automations you write for a window AC work the same way for a mini-split or whole-house HVAC

- The presence detection logic is universal — it doesn't care how big the system is

- The battery management principles are the same whether you have a $500 EcoFlow or a $5,000 EG4 with LiFePO4 rack batteries

Start here. Build the skills. When you have a house with a roof, you already know exactly what to do — at ten times the scale.

The system I run on Maui — a full EG4 18kPV solar-plus-battery setup with fire sprinkler automation, NASA FIRMS fire detection, and whole-property energy management — runs on the same Home Assistant foundation you'd use to manage a window AC and a EcoFlow in a one-bedroom apartment. Same logic. Same tools. Different numbers.

Next post: Setting up your first EMS automation in Home Assistant — reading your solar production sensor and writing your first threshold-based load trigger. No coding required.

ExpertAmateur.com  |  Short Video Script  |  ~2 minutes

# VIDEO SCRIPT: The Apartment Energy Stack

Balcony Solar + Battery + EMS + Presence Detection

HOOK (0:00–0:12)

[Tight shot: window AC unit running. Cut to: solar panel on balcony in bright sun. Cut to: phone showing Home Assistant dashboard. Quick cuts, energetic.]

Balcony solar alone is passive. It makes power when the sun shines and hopes you're using something. That's not a system — that's a coincidence.

This is what a real apartment energy system looks like.

THE PROBLEM WITH PASSIVE (0:12–0:30)

[Simple graphic: sun hits panel → power goes... nowhere useful. Clock showing noon vs. 7pm.]

Here's the problem. Peak solar is midday. Peak apartment energy use is morning and evening. Without storage, you're missing most of your own production — it either exports at low or zero credit, or just doesn't match when you need it.

You could be capturing 80, 90 percent of what your panel produces. Instead you're getting maybe 60.

THE FOUR-LAYER STACK (0:30–0:90)

[Animated stack diagram building layer by layer as each is mentioned]

Layer one: the panel. One or two 400-watt bifacial panels on your balcony or railing. Your generation source.

Layer two: a battery. A portable LiFePO4 power station — EcoFlow, Bluetti, something in the one to two kilowatt-hour range. Now you can store what the panel makes and use it when you actually need it.

Layer three: the EMS — Energy Management System. Home Assistant on a Raspberry Pi. This watches your solar production in real time and makes decisions. When output exceeds your baseline loads, it charges the battery. When you're in time-of-use peak hours, it draws from storage instead of the grid. When solar is strong and you're home, it runs your big loads now — not tonight.

Layer four: presence detection. This is what makes it genuinely intelligent.

[Cut to: phone GPS showing someone leaving home. HA automation fires — AC turns off.]

Your phone tells Home Assistant when you leave. The system shuts the AC off immediately. No schedule. No timer. You left, so it stopped. When you're ten minutes from home, it starts pre-cooling using stored solar — so the apartment is comfortable when you walk in. Peak rates, no grid draw.

[Cut to: Bill on camera]

A Nest or Ecobee does some of this. But it doesn't know about your solar production or your battery state. It's optimizing for comfort. Your EMS optimizes for comfort and energy source simultaneously. That's a different category of smart.

THE NUMBERS (1:30–1:45)

[Simple on-screen text: passive system vs. managed system comparison]

A passive $1,000 balcony system might save you $120 a year. The same hardware with an EMS and presence detection — realistically $150 or more, because you're capturing production you'd otherwise miss. That's a meaningful compression of your payback window, and it compounds as electricity rates climb.

THE LADDER (1:45–1:55)

[Three-rung ladder graphic: Apartment Stack → Add Storage → Full Property]

This is Tier One of the ExpertAmateur energy path. Every automation you build here scales directly to a larger system. The skills transfer completely.

CLOSE (1:55–2:05)

[Bill on camera, casual. ExpertAmateur logo lower third.]

Full build guide, hardware list, and your first Home Assistant automation — linked below. I'm Bill at ExpertAmateur. Build smarter.

[End card: subscribe, blog post link, next video teaser]